MEMORY ITEMS FOR POWER LOSS IN TWIN ENGINE ARIPLANES

- MIXTURES			(Full forward)

- PROPS				(Full forward)

- THROTTLES			(Full forward)

- FLAPS				Up

- GEAR				Up

- IDENTIFY			“Dead foot, dead engine”

- VERIFY				Retard suspect throttle

- FIX

  - If 3,000 AGL or higher and no catastrophic failure then go through flows to try to fix. For a B55 Baron you:

    - Check fuel selectors (consider cross feed)

    - Mixtures forward

    - Consider using electric fuel pump

    - Cycle the magneto

- OR FEATHER

  - If below 3,000 AGL:

    - Feather the propeller (Inoperative engine)

  - When simulating an engine failure the Pilot in Training (PT) should use a "karate chop" to identify the engine to be feathered (along with verbalizing, e.g. "I would feather the (right or left) engine". This reassures the instructor that the PT will not inadvertently feather the prop on an operating engine.

- RAISE THE DEAD ENGINE

- CHECKLIST

You can teach, or use, this by saying (and doing) "Mixture, Props, Throttles, full forward. Gear and Flaps up. Identify - dead foot, dead engine. Verify, Fix or Feather. Raise the Dead engine (2-5 degrees), split the ball on the good side, pitch for blue line. Break out the checklist"

-Remember, no fast hands in the cockpit. You do not want to act in haste and inadvertently pull a lever that you did not mean to pull. "Slow is smooth, smooth is fast."