Takeoff Briefing

ENGINE FAILURE DURING TAKEOFF ROLL

Abort, Abort, Abort (Anyone can call it). Throttle to Idle. Max Brakes. Maintain Directional Control. Broadcast your Intentions

ENGINE FAILURE AFTER ROTATION WITH RUNWAY REMAINING

Throttle to Idle. Land Straight Ahead. Max Brakes. Maintain Directional Control.

Broadcast Your Intentions

ENGINE FAILURE AFTER ROTATION WITHOUT RUNWAY REMAINING (Below 1,000' AGL)

Throttle to idle. Pitch for 70 Knots. Find an Alternate Landing Area Straight Ahead (up to +/- 450 off Nose).

Flaps as Required. If Time Permits, Secure the Engine Before Landing. Broadcast your Intentions.

ENGINE FAILURE ABOVE 1,000' AGL

Pitch for 70 Knots. Time Permitting we will Attempt to Return and Land at the Airport and Broadcast our Intentions. Flaps as required. Otherwise We Will Find Another Suitable Landing Site.

In the Event of an Emergency I will Fly, you Follow  me on the Checklist

Any Questions?

Notes:

- Airspeeds are aircraft dependent. Speeds above are for C182Q.

- Adjust altitudes as necessary, turning back before 1,000ft should only be done if properly trained, maneuver has been practiced and pilot has knowledge of what altitude that he/she can safely turn back towards the airport.

- Adjust procedures as necessary to be appropriate for your aircraft.