S.A.F.E.T.Y.

S ignal – always broadcast ADS-B signal

A TIS/Avoid – monitor the ATIS at RDM and avoid the active runway final approach course

F light following for all practice approach operations to active runway

E ars tuned in to always monitor Seattle Center frequency (126.5)

T ransitions through the active RWY final approach course need coordination with ATC

Y our efforts help keep central Oregon skies SAFE! Thank you!

- ALWAYS broadcast ADS-B

- ALWAYS monitor the ATIS at RDM and remain clear of the final approach course to the active RWY.

  - RWY 5 in use implies that all aircraft should remain clear of the RWY 5 corridor (shared in the presentation and available in KML format/Foreflight).

  - RWY 23, 29, or 11 in use implies that aircraft should remain clear of the respective final approach course to that active runway.  This means that if RWY 5 is not the active/advertised RWY, you are free to maneuver within the RWY 5 corridor without prior coordination or ATC notification (however, please still monitor the frequency).

- If you would like to execute a practice approach to the advertised runway, please call for flight following.  If flight following is unavailable, continue to monitor the freq. and try again in 5 mins.

- ALWAYS tune in and monitor the frequency (126.15)

- To transition through the active RWY’s final approach course when you do not want or are unable to receive flight following, please call ATC to coordinate your transition.

Pilot: “N1234 request corridor transition.”

ATC’s response will be one of the following:

- “No IFR traffic in corridor”  - this implies that you may transition the corridor

- “Traffic in corridor, advise you remain clear.”  Please remain clear of the corridor. This indicates that the controller believes there is a potential conflict with IFR aircraft or may be too busy to provide more detailed information about the potential traffic in the area.

- Workload permitting, ATC may advise you of where the traffic is within the corridor, i.e.: “DASH8 traffic, ZIBID, inbound RNAV Y runway 5.” This allows you to use best judgement as to whether or not you can transition the corridor without creating a conflict with the inbound aircraft.  Please keep in mind that “see and avoid” is not adequate in this situation.  Please allow a generous amount of space between you and the aircraft on approach in order to ensure that a TCAS RA is not triggered and potentially causes the aircraft to deviate from the altitude and/or route on the approach procedure.